Schedule of Condition

Of

parts of

{{ao_property_add_vert}}

Created for

{{bo_letter_names}}

and

{{ao_letter_names}}

This Schedule is limited to those areas of the building in the immediate vicinity of planned works at

{{bo_property_add_horz}}

27th January 2021

Section A – General Information

{{bo_plural}}: {{bo_notice_names}} {{bo_apostrophe}} Address: {{bo_property_add_horz}} {{ao_plural}}: {{ao_notice_names}} {{ao_apostrophe}} Address: {{ao_property_add_horz}}

Date of Inspection: 27th January 2021

Section B – General Summary

| Type of property: Weather conditions at time of inspection: The state of the property when inspected: | Residential. Overcast and damp. Occupied, fully furnished. |
| --- | --- |

Section C – Condition

| GENERAL NOTES: |  | CRACK WIDTHS: |
| --- | --- | --- |
| Object Descriptions: - All directions assume facing the object being described. References to walls in this schedule are as follows:- Front Wall = Wall nearest the road. Rear Wall = Wall furthest from the road. Left Wall = Wall on the left-hand side when oriented from the road. Right Wall = Wall on the right-hand side when oriented from the road. |  | Hairline: <= 0.1mm Fine: 0.2mm – 2mm Moderate: 2mm – 5mm Serious: 5mm – 15mm Severe: 15mm – 25mm Very Severe: >25mm |

The following limitations apply to the conduct of the inspection:-

- The inspection was purely of a visual nature, without the use of ladders or the benefit of exploratory work. Its extent is limited to cracks to internal walls, floors and ceilings and significant defects to exterior walls, roof and boundaries, having regard to their age and construction type.

- No structural survey or inspection of concealed or otherwise hidden services or finishes was undertaken.

- The presence of furniture, fittings, furnishings, artwork and stored items generally restricts the scope of inspection, and where such articles or items exist, none were moved. Ceiling, wall and floors surfaces concealed by such articles or items were not inspected.

- No testing of any service installations was undertaken.

- All cracks are of hair width unless otherwise stated.

- This written Schedule is to be read in conjunction with the photographs taken at the same time.

Photographic Record Notes:

- Photo of Two Fingers: Indicates defect in the door/window/floor pointed at. Not used to indicate cracks.

- Photo of Single Finger: Indicates slope in floor moving down in the direction being pointed towards.

- Thumbs Up: No defect noted with the object in the background. Usually used to indicate windows and/or doors open and close freely.

- Crossed Fingers: Unable to open door or window pointed at.

- Bent finger: Used to show a bulge (convex finger) or dip (concave finger) in the floor.

EXTERNAL OBSERVATIONS

Rear

- Photographic record taken.

- The garden wall is leaning significantly towards the Building Owners’ side and does not follow a straight line.

Front

- Photographic record taken.

INTERNAL OBSERVATIONS

17 St Marys

Ground Floor

Rear Bedroom

- Survey starts at door and moves around to the right.

- Detailed survey taken of left hand half of room, photographic record taken of right hand half of room.

- Door opens and closes without binding.

- Vertical hairline crack through cornice on rear wall located the above left-hand side of window.

- Left hand window binds against frame in bottom right corner.

- Top window opens and closes without binding.

- Unable to open right hand window as the handle is detached.

- Vertical hairline crack on rear wall starting at window sill located 200mm from left hand side of window sill moving down to skirting.

Hallway

- Survey starts at rear bedroom and moves forwards.

- Open cornice mitre joint in rear left corner above left-hand side of bedroom doorway.

- Hairline crack in ceiling running from left to right hand walls located 3m from rear bedroom.

- Curved vertical fine crack on left hand wall starting 50mm below cornice and 200mm from a dogleg projection in the left hand wall opposite the bathroom moving down for 500mm before moving diagonally down and to the left for a further 200mm.

- Open joint between and dogleg stepped in section of left hand wall and cornice, continuing vertically as open cornice mitre joint.

- Two parallel hairline cracks in ceiling running from left to right hand walls at the corner of hallway adjacent to bathroom.

- Open cornice mitre joint at corner of bathroom partition wall.

- Photographic record of the remainder of hallway where it steps in at stairwell to 1st floor and continuing to front entrance door.

17A St Marys

Ground Floor Entrance Hallway and Stairs between Ground and First Floor

- Photographic record taken.

First Floor

Kitchen

- Detailed survey taken of left hand half of room, photographic record taken of right hand half of room.

- Survey starts at hallway door and moves around to the right.

- Hallway door binds slightly against frame on right hand side.

- Vertical hairline crack on front wall starting at top of the hallway doorframe located 120mm right of top left corner of doorframe moving up to high level box detail.

- Vertical hairline crack on front wall starting at top of hallway doorframe located 120mm left of top right corner of hallway doorframe moving up to high level box detail.

- Open joints to high level box detail between abutting surfaces and dirt sections forming the box detail itself.

- Fine open joint narrowing to hairline between ceiling and high level box detail extending full width.

- Open joint (hairline to 2 mm) between ceiling and high level box detail along left hand wall extending full length.

- Hairline crack on ceiling starting at high level box detail located above left-hand side of hob extractor moving towards right hand wall for 300mm.

- Multiple small pockmarks in plastered finish of left hand wall below boiler with multiple light hairline cracks in same area and likewise above the boiler on left hand wall.

- Open joint between ceiling and rear wall extending full length.

- Left hand window does not open, the latch has failed. The right hand window binds against frame in bottom left corner.

- Intermittent open joint between window frame and top reveal extending full width.

- The worktop and along the rear wall has dropped by approximately 10mm with maximum separation located behind sinks.

- External door binds against sill in bottom left corner.

- Floor slopes down from left to right and in front half of the room and slopes down towards the middle.

Hallway

- Photographic record taken as this is set back slightly from the party wall.

- Survey starts at kitchen doorway and moves forwards.

- Top left door to built-in cupboard and does not open.

- Top right corner of cupboard opens and closes without binding.

- The two bottom doors to built-in cupboard are securely shut and do not open.

- Rucking of wallpaper and faint open joint at junction of built in cupboard and left hand wall extending full height.

Bathroom

- Survey starts at door and moves round to the right.

- Door opens and closes without binding although creaks when open.

- Open joint between right hand side of doorframe and right hand wall extending full height of doorframe.

- Open joint between ceiling and rear wall extending full width.

- Open joint between tiled section of right rear wall and doorframe intermittently all round.

- Open joint between tiled rear wall and bath side panel extending full height of bath.

- Compression fracture to right hand wall where the door handle has been impacting, two patch repairs in the vicinity.

- Two staggered vertical hairline cracks on right hand wall emerging from behind radiator located left of centre of radiator moving up for 1m above radiator.

- Open joint between ceiling and timber facia to which the curtain rail is fixed extending full width.

- Open joint between ceiling and box detail in front left corner.

- Window is stiff to open.

- Vertical hairline crack on front wall starting at window sill located 600mm from right hand window reveal moving down to skirting board.

- Horizontal open joint between timber panelled sections of box detail starting at right hand side of window sill moving to left hand wall.

- Curved horizontal hairline crack on rear face of timber box detail located 500mm below ceiling moving to left hand wall, continuing along left-hand wall moving in a curved fashion terminating at the sixth tile on the top course is measured from the timber box detail in front left corner of room.

- Open joint between ceiling and left hand wall starting at front wall moving to tiled section of left hand wall.

- Paint peeling on front wall adjacent to left hand wall and directly above skirting board.

- Floor slopes down from right to left.

Stairs between First and Second Floor

- S shaped hairline crack on left hand wall starting at junction of first floor ceiling and vertical upstand/2nd floor riser moving forwards for 400mm.

- Multiple areas of patch repair and lifted wallpaper to left hand wall running adjacent and below the rear sloping ceiling for full length.

- Open joint between rear sloping ceiling and rear wall starting at left hand wall moving to right hand wall with a hairline crack projecting into the ceiling running parallel to rear wall by between 15mm and 20mm extending the majority of the width of this wall.

- Windows open and close without binding.

Second Floor

Toilet

- Photographic record taken.

- Survey starts at door and moves around to the right.

- Door binds against carpet.

Store Room

- Photographic record taken.

- Unable to access built in cupboards on landing.